- Easily Interfaced to Microprocessors

- On-Chip Data Latches

- Monotonic Over the Entire A/D Conversion Range

- Segmented High-Order Bits Ensure Low-Glitch Output

- Interchangeable With Analog Devices AD7524, PMI PM-7524, and Micro Power Systems MP7524

- Fast Control Signaling for Digital

Signal-Processor Applications Including Interface With TMS320

3    2    1   20 19

CMOS Technology

D, N, OR PW PACKAGE (TOP VIEW)

| OUT1 | 1 | 16 | RFB |
| --- | --- | --- | --- |
| OUT2 | 2 | 15 | REF |
| GND | 3 | 14 | VDD |
| DB7 | 4 | 13 | WR |
| DB6 | 5 | 12 | CS |
| DB5 | 6 | 11 | DB0 |
| DB4 | 7 | 10 | DB1 |
| DB3 | 8 | 9 | DB2 |

OUT2 OUT1 NC RFB REF

FN PACKAGE (TOP VIEW)

| KEY PERFORMANCE SPECIFICATIONS |  |
| --- | --- |
| Resolution | 8 Bits |
| Linearity error | 1/2LSB Max |
| Power dissipation at VDD = 5V Setting time | 5mW Max 100ns Max |
| Propagation delay time | 80ns Max |

description

The TLC7524C, TLC7524E, and TLC7524I   are

CMOS, 8-bit, digital-to-analog converters (DACs) designed for easy interface to most popular microprocessors.

DB4 DB3 NC DB2 DB1

| GND | 4 | 18 |  | VDD |
| --- | --- | --- | --- | --- |
| DB7 | 5 | 17 |  | WR |
| NC | 6 | 16 |  | NC |
| DB6 | 7 | 15 |  | CS |
| DB5 | 8 | 14 |  | DB0 |
|  | 9 10 1 | 1 12 13 |  |  |

NC−No internal connection

The devices are 8-bit, multiplying DACs with input latches and load cycles similar to the write cycles of a random access memory. Segmenting the high-order bits minimizes glitches during changes in the most significant bits, which produce the highest glitch impulse. The devices provide accuracy to 1/2LSB without the need for thin-film resistors or laser trimming, while dissipating less than 5mW typically.

Featuring operation from a 5V to 15V single supply, these devices interface easily to most microprocessor buses or output ports. The 2- or 4-quadrant multiplying makes these devices an ideal choice for many microprocessor-controlled gain-setting and signal-control applications.

The TLC7524C is characterized for operation from 0C to 70C. The TLC7524I is characterized for operation from −25C to +85C. The TLC7524E is characterized for operation from − 40C to +85C.

Please be aware that an important notice concerning availability, standard warranty, and use in  critical applications of Texas Instruments semiconductor products and disclaimers thereto appears at the end of this data sheet.

All trademarks are the property of their respective owners.

PRODUCTION   DATA   information  is  current  as  of   publication  date.	Copyright  1998−2007, Texas Instruments Incorporated

Products conform to specifications per the terms of Texas Instruments standard warranty. Production processing does not necessarily include testing of all parameters.

##### functional block diagram

15

R	R	R

REF

2R

S-1

2R

S-2

2R

S-3

2R	2R

S-8	R

16

RFB

- OUT1

- OUT2

12

CS

13

WR

Data Latches

- GND

4

DB7 (MSB)

5

DB6

6

DB5

11

DB0 (LSB)

Data Inputs

Terminal numbers shown are for the D or N package.

##### absolute maximum ratings over operating free-air temperature range (unless otherwise noted)

Supply voltage range, VDD

. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .   −0.3V to 16.5V

Digital input voltage range, VI   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .  −0.3V to VDD + 0.3V

Reference voltage, Vref	. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .

 25V

Peak digital input current, II   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .  10A

Operating free-air temperature range, TA:   TLC7524C	. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .  0C to +70C

TLC7524I	. . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .   −25C to +85C

TLC7524E	. . . . . . . . . . . . . . . . . . . . . . . . . . . . . .   −40C to +85C

Storage temperature range, Tstg   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .   −65C to +150C

Case temperature for 10 seconds, TC: FN package   . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . . .

+260C

Lead temperature 1,6mm (1/16 inch) from case for 10 seconds: D, N, or PW package

package/ordering information

. . . . . . . . . . .

+260C

For the most current package and ordering information, see the Package Option Addendum at the end of this document, or see the TI website at www.ti.com.

##### recommended operating conditions

|  |  | VDD = 5V | VDD = 15V | UNIT |
| --- | --- | --- | --- | --- |
|  |  | MIN NOM MAX | MIN NOM MAX |  |
| Supply voltage, VDD |  | 4.75 5 5.25 | 14.5 15 15.5 | V |
| Reference voltage, Vref |  |  10 |  10 | V |
| High-level input voltage, VIH |  | 2.4 | 13.5 | V |
| Low-level input voltage, VIL |  | 0.8 | 1.5 | V |
| CS setup time, tsu(CS) |  | 40 | 40 | ns |
| CS hold time, th(CS) |  | 0 | 0 | ns |
| Data bus input setup time, tsu(D) |  | 25 | 25 | ns |
| Data bus input hold time, th(D) |  | 10 | 10 | ns |
| Pulse duration, WR low, tw(WR) |  | 40 | 40 | ns |
| Operating free-air temperature, TA | TLC7524C | 0 +70 | 0 +70 | C |
|  | TLC7524I | −25 +85 | −25 +85 |  |
|  | TLC7524E | −40 +85 | −40 +85 |  |

electrical characteristics over recommended operating free-air temperature range, Vref = 10V, OUT1 and OUT2 at GND (unless otherwise noted)

| PARAMETER |  | TEST CONDITIONS | VDD = 5V | VDD = 15V | UNIT |
| --- | --- | --- | --- | --- | --- |
|  |  |  | MIN TYP MAX | MIN TYP MAX |  |
| IIH High-level input current |  | VI = VDD | 10 | 10 | A |
| IIL Low-level input current |  | VI = 0 | −10 | −10 | A |
| IIkg Output leakage current | OUT1 | DB0−DB7 at 0V, WR, CS at 0V, Vref =  10V |  400 |  200 | nA |
|  | OUT2 | DB0−DB7 at VDD, WR, CS at 0V, Vref =  10V |  400 |  200 |  |
| IDD Supply current | Quiescent | DB0−DB7 at VIHmin or VILmax | 1 | 2 | mA |
|  | Standby | DB0−DB7 at 0V or VDD | 500 | 500 | A |
| kSVS Supply voltage sensitivity, gain/VDD |  | VDD =  10% | 0.01 0.16 | 0.005 0.04 | %FSR/% |
| Ci Input capacitance, DB0−DB7, WR, CS |  | VI = 0 | 5 | 5 | pF |
| Co Output capacitance | OUT1 | DB0−DB7 at 0V, WR, CS at 0V | 30 | 30 | pF |
|  | OUT2 |  | 120 | 120 |  |
|  | OUT1 | DB0−DB7 at VDD, WR, CS at 0V | 120 | 120 |  |
|  | OUT2 |  | 30 | 30 |  |
| Reference input impedance (REF to GND) |  |  | 5 20 | 5 20 | k |

operating characteristics over recommended operating free-air temperature range, Vref = 10V, OUT1 and OUT2 at GND (unless otherwise noted)

| PARAMETER | TEST CONDITIONS | VDD = 5V | VDD = 15V | UNIT |
| --- | --- | --- | --- | --- |
|  |  | MIN TYP MAX | MIN TYP MAX |  |
| Linearity error |  |  0.5 |  0.5 | LSB |
| Gain error | See Note 1 |  2.5 |  2.5 | LSB |
| Settling time (to 1/2 LSB) | See Note 2 | 100 | 100 | ns |
| Propagation delay from digital input to 90% of final analog output current | See Note 2 | 80 | 80 | ns |
| Feedthrough at OUT1 or OUT2 | Vref = 10V (100kHz sinewave) WR and CS at 0V, DB0−DB7 at 0V | 0.5 | 0.5 | %FSR |
| Temperature coefficient of gain | TA = +25C to MAX |  0.004 |  0.001 | %FSR/C |

NOTES: 1.  Gain error is measured using the internal feedback resistor. Nominal full-scale range (FSR) = Vref − 1LSB.

2.   OUT1 load = 100, Cext = 13pF, WR at 0V, CS at 0V, DB0 − DB7 at 0V to VDD or VDD to 0V.

##### operating sequence

tsu(CS)

CS

th(CS)

tw(WR)

WR

tsu(D)

th(D)

DB0−DB7

voltage-mode operation

It is possible to operate the current-multiplying DAC in these devices in a voltage mode. In the voltage mode, a fixed voltage is placed on the current output terminal. The analog output voltage is then available at the reference voltage terminal. Figure 1 is an example of a current-multiplying DAC, which is operated in voltage mode.

2R

2R

2R

2R

0

1

R

R	R	R

REF (Analog Output Voltage)

OUT1 (Fixed Input Voltage) OUT2

Figure 1. Voltage Mode Operation

The relationship between the fixed-input voltage and the analog-output voltage is given by the following equation:

where

VO = VI (D/256)

VO = analog output voltage VI   = fixed input voltage

D   = digital input code converted to decimal

In voltage-mode operation, these devices meet the following specification:

| PARAMETER | TEST CONDITIONS |  |  |  | MIN MAX | UNIT |
| --- | --- | --- | --- | --- | --- | --- |
| Linearity error at REF | VDD = 5V, | OUT1 = 2.5V, | OUT2 at GND, | TA = +25C | 1 | LSB |

The TLC7524C, TLC7524E, and TLC7524I are 8-bit multiplying DACs consisting of an inverted R-2R ladder, analog switches, and data input latches. Binary-weighted currents are switched between the OUT1 and OUT2 bus lines, thus maintaining a constant current in each ladder leg independent of the switch state. The high-order bits are decoded. These decoded bits, through a modification in the R-2R ladder, control three equally-weighted current sources. Most applications only require the addition of an external operational amplifier and a voltage reference.

The equivalent circuit for all digital inputs low is seen in Figure 2. With all digital inputs low, the entire reference current, Iref, is switched to OUT2. The current source I/256 represents the constant current flowing through the termination resistor of the R-2R ladder, while the current source IIkg represents leakage currents to the substrate. The capacitances appearing at OUT1 and OUT2 are dependent upon the digital input code. With all digital inputs high, the off-state switch capacitance (30pF maximum) appears at OUT2 and the on-state switch capacitance (120pF maximum) appears at OUT1. With all digital inputs low, the situation is reversed as shown in Figure 2. Analysis of the circuit for all digital inputs high is similar to Figure 2; however, in this case, Iref would be switched to OUT1.

The DAC on these devices interfaces to a microprocessor through the data bus and the CS and WR control signals. When CS and WR are both low, analog output on these devices responds to the data activity on the DB0−DB7 data bus inputs. In this mode, the input latches are transparent and input data directly affects the analog output. When either the CS signal or WR signal goes high, the data on the DB0−DB7 inputs are latched until the CS and WR signals go low again. When CS is high, the data inputs are disabled regardless of the state of the WR signal.

These devices are capable of performing 2-quadrant or full 4-quadrant multiplication. Circuit configurations for 2-quadrant or 4-quadrant multiplication are shown in Figure 3 and Figure 4. Table 1 and Table 2 summarize input coding for unipolar and bipolar operation respectively.

R

30 pF

RFB

IIkg

OUT1

Iref

I/256

IIkg

120 pF

REF	OUT2

Figure 2. TLC7524 Equivalent Circuit With All Digital Inputs Low

RA = 2 k (see Note A)

RB

RFB

C (see Note B)

OUT1

−

OUT2

+

GND

Vref	VDD

DB0−DB7

CS WR

Output

NOTES:  A.  RA and RB used only if gain adjustment is required.

B.       C phase compensation (10-15 pF) is required when using high-speed amplifiers to prevent

ringing or oscillation.

Figure 3. Unipolar Operation (2-Quadrant Multiplication)

RA = 2 k (see Note A)

DB0−DB7

CS WR

Vref

VDD

GND

RB

RFB

OUT1 OUT2

C (see Note B)

−

+

20 k

10 k

20 k

−

+

5 k

Output

NOTES:  A.  RA and RB used only if gain adjustment is required.

B.  C phase compensation (10-15 pF) is required when using high-speed amplifiers to prevent ringing or oscillation.

Figure 4. Bipolar Operation (4-Quadrant Operation)

| DIGITAL INPUT (see Note 3) | ANALOG OUTPUT |
| --- | --- |
| MSB LSB |  |
| 1 1 1 1 1 1 1 1 | − Vref (255/256) |
| 1 0 0 0 0 0 0 1 | − Vref (129/256) |
| 1 0 0 0 0 0 0 0 | − Vref (128/256) = −Vref/2 |
| 0 1 1 1 1 1 1 1 | − Vref (127/256) |
| 0 0 0 0 0 0 0 1 | − Vref (1/256) |
| 0 0 0 0 0 0 0 0 | 0 |

| DIGITAL INPUT (see Note 4) | ANALOG OUTPUT |
| --- | --- |
| MSB LSB |  |
| 1 1 1 1 1 1 1 1 | Vref (127/128) |
| 1 0 0 0 0 0 0 1 | Vref (1/128) |
| 1 0 0 0 0 0 0 0 | 0 |
| 0 1 1 1 1 1 1 1 | − Vref (1/128) |
| 0 0 0 0 0 0 0 1 | − Vref (127/128) |
| 0 0 0 0 0 0 0 0 | − Vref |

Table 1.  Unipolar Binary Code	Table 2.  Bipolar (Offset Binary) Code

NOTE 3:   LSB = 1/256 (Vref)	NOTE 4:  LSB = 1/128 (Vref)

##### microprocessor interfaces

D0−D7

Z−80A

WR

IORQ

A0−A15

Decode Logic

Data Bus

WR

DB0−DB7

TLC7524

CS

OUT1

OUT2

Address Bus

Figure 5. TLC7524: Z-80A Interface

D0−D7 6800

2

VMA

A0−A15

Decode Logic

Data Bus

WR

DB0−DB7

TLC7524

CS

OUT1

OUT2

Address Bus

Figure 6. TLC7524: 6800 Interface

##### PRINCIPLES OF OPERATION

microprocessor interfaces (continued)

Decode Logic

A8−A15

8051

ALE

WR

AD0−AD7

8-Bit Latch

Address Bus

CS

OUT1

WR	TLC7524

OUT2

DB0−DB7

Adress/Data Bus

Figure 7. TLC7524: 8051 Interface

Revision History

| DATE | REV | PAGE | SECTION | DESCRIPTION |
| --- | --- | --- | --- | --- |
| 6/07 | D | Front Page | — | Deleted Available Options table. |
|  |  | 2 | — | Inserted Package/Ordering information. |

NOTE:  Page numbers for previous revisions may differ from page numbers in the current version.

### PACKAGE OPTION ADDENDUM

www.ti.com	11-Nov-2025

##### PACKAGING INFORMATION

| Orderable part number | Status Material type (1) (2) |  | Package ¦ Pins | Package qty ¦ Carrier | RoHS (3) | Lead finish/ Ball material (4) | MSL rating/ Peak reflow (5) | Op temp (°C) | Part marking (6) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TLC7524CD | Active | Production | SOIC (D) ¦ 16 | 40 ¦ TUBE | Yes | NIPDAU | Level-1-260C-UNLIM | 0 to 70 | TLC7524C |
| TLC7524CD.A | Active | Production | SOIC (D) ¦ 16 | 40 ¦ TUBE | Yes | NIPDAU | Level-1-260C-UNLIM | 0 to 70 | TLC7524C |
| TLC7524CDG4 | Active | Production | SOIC (D) ¦ 16 | 40 ¦ TUBE | Yes | NIPDAU | Level-1-260C-UNLIM | 0 to 70 | TLC7524C |
| TLC7524CDR | Active | Production | SOIC (D) ¦ 16 | 2500 ¦ LARGE T&R | Yes | NIPDAU | Level-1-260C-UNLIM | 0 to 70 | TLC7524C |
| TLC7524CDR.A | Active | Production | SOIC (D) ¦ 16 | 2500 ¦ LARGE T&R | Yes | NIPDAU | Level-1-260C-UNLIM | 0 to 70 | TLC7524C |
| TLC7524CFNR | Obsolete | Production | PLCC (FN) ¦ 20 | - | - | Call TI | Call TI | 0 to 70 | TLC7524C |
| TLC7524CN | Active | Production | PDIP (N) ¦ 16 | 25 ¦ TUBE | Yes | NIPDAU | N/A for Pkg Type | 0 to 70 | TLC7524CN |
| TLC7524CN.A | Active | Production | PDIP (N) ¦ 16 | 25 ¦ TUBE | Yes | NIPDAU | N/A for Pkg Type | 0 to 70 | TLC7524CN |
| TLC7524CNE4 | Active | Production | PDIP (N) ¦ 16 | 25 ¦ TUBE | Yes | NIPDAU | N/A for Pkg Type | 0 to 70 | TLC7524CN |
| TLC7524CNS | Active | Production | SOP (NS) ¦ 16 | 50 ¦ TUBE | Yes | NIPDAU | Level-1-260C-UNLIM | 0 to 70 | TLC7524 |
| TLC7524CNS.A | Active | Production | SOP (NS) ¦ 16 | 50 ¦ TUBE | Yes | NIPDAU | Level-1-260C-UNLIM | 0 to 70 | TLC7524 |
| TLC7524CNSR | Active | Production | SOP (NS) ¦ 16 | 2000 ¦ LARGE T&R | Yes | NIPDAU | Level-1-260C-UNLIM | 0 to 70 | TLC7524 |
| TLC7524CNSR.A | Active | Production | SOP (NS) ¦ 16 | 2000 ¦ LARGE T&R | Yes | NIPDAU | Level-1-260C-UNLIM | 0 to 70 | TLC7524 |
| TLC7524CPW | Active | Production | TSSOP (PW) ¦ 16 | 90 ¦ TUBE | Yes | NIPDAU | Level-1-260C-UNLIM | 0 to 70 | P7524 |
| TLC7524CPW.A | Active | Production | TSSOP (PW) ¦ 16 | 90 ¦ TUBE | Yes | NIPDAU | Level-1-260C-UNLIM | 0 to 70 | P7524 |
| TLC7524CPWR | Active | Production | TSSOP (PW) ¦ 16 | 2000 ¦ LARGE T&R | Yes | NIPDAU | Level-1-260C-UNLIM | 0 to 70 | P7524 |
| TLC7524CPWR.A | Active | Production | TSSOP (PW) ¦ 16 | 2000 ¦ LARGE T&R | Yes | NIPDAU | Level-1-260C-UNLIM | 0 to 70 | P7524 |
| TLC7524ED | Active | Production | SOIC (D) ¦ 16 | 40 ¦ TUBE | Yes | NIPDAU | Level-1-260C-UNLIM | -40 to 85 | TLC7524E |
| TLC7524ED.A | Active | Production | SOIC (D) ¦ 16 | 40 ¦ TUBE | Yes | NIPDAU | Level-1-260C-UNLIM | -40 to 85 | TLC7524E |
| TLC7524EDR | Active | Production | SOIC (D) ¦ 16 | 2500 ¦ LARGE T&R | Yes | NIPDAU | Level-1-260C-UNLIM | -40 to 85 | TLC7524E |
| TLC7524EDR.A | Active | Production | SOIC (D) ¦ 16 | 2500 ¦ LARGE T&R | Yes | NIPDAU | Level-1-260C-UNLIM | -40 to 85 | TLC7524E |
| TLC7524EN | Active | Production | PDIP (N) ¦ 16 | 25 ¦ TUBE | Yes | NIPDAU | N/A for Pkg Type | -40 to 85 | TLC7524EN |
| TLC7524EN.A | Active | Production | PDIP (N) ¦ 16 | 25 ¦ TUBE | Yes | NIPDAU | N/A for Pkg Type | -40 to 85 | TLC7524EN |
| TLC7524ID | Active | Production | SOIC (D) ¦ 16 | 40 ¦ TUBE | Yes | NIPDAU | Level-1-260C-UNLIM | -25 to 85 | TLC7524I |
| TLC7524ID.A | Active | Production | SOIC (D) ¦ 16 | 40 ¦ TUBE | Yes | NIPDAU | Level-1-260C-UNLIM | -25 to 85 | TLC7524I |
| TLC7524IDR | Active | Production | SOIC (D) ¦ 16 | 2500 ¦ LARGE T&R | Yes | NIPDAU | Level-1-260C-UNLIM | -25 to 85 | TLC7524I |
| TLC7524IDR.A | Active | Production | SOIC (D) ¦ 16 | 2500 ¦ LARGE T&R | Yes | NIPDAU | Level-1-260C-UNLIM | -25 to 85 | TLC7524I |
| TLC7524IFN | Obsolete | Production | PLCC (FN) ¦ 20 | - | - | Call TI | Call TI | -25 to 85 | TLC7524I |
| TLC7524IN | Active | Production | PDIP (N) ¦ 16 | 25 ¦ TUBE | Yes | NIPDAU | N/A for Pkg Type | -25 to 85 | TLC7524IN |

Addendum-Page 1

### PACKAGE OPTION ADDENDUM

www.ti.com	11-Nov-2025

| Orderable part number | Status (1) | Material type (2) | Package ¦ Pins | Package qty ¦ Carrier | RoHS (3) | Lead finish/ Ball material (4) | MSL rating/ Peak reflow (5) | Op temp (°C) | Part marking (6) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TLC7524IN.A | Active | Production | PDIP (N) ¦ 16 | 25 ¦ TUBE | Yes | NIPDAU | N/A for Pkg Type | -25 to 85 | TLC7524IN |
| TLC7524IPW | Active | Production | TSSOP (PW) ¦ 16 | 90 ¦ TUBE | Yes | NIPDAU | Level-1-260C-UNLIM | -25 to 85 | Y7524 |
| TLC7524IPW.A | Active | Production | TSSOP (PW) ¦ 16 | 90 ¦ TUBE | Yes | NIPDAU | Level-1-260C-UNLIM | -25 to 85 | Y7524 |
| TLC7524IPWR | Active | Production | TSSOP (PW) ¦ 16 | 2000 ¦ LARGE T&R | Yes | NIPDAU | Level-1-260C-UNLIM | -25 to 85 | Y7524 |
| TLC7524IPWR.A | Active | Production | TSSOP (PW) ¦ 16 | 2000 ¦ LARGE T&R | Yes | NIPDAU | Level-1-260C-UNLIM | -25 to 85 | Y7524 |

(1) Status:  For more details on status, see our product life cycle.

(2) Material type: When designated, preproduction parts are prototypes/experimental devices, and are not yet approved or released for full production. Testing and final process, including without limitation quality assurance, reliability performance testing, and/or process qualification, may not yet be complete, and this item is subject to further changes or possible discontinuation. If available for ordering, purchases will be subject to an additional waiver at checkout, and are intended for early internal evaluation purposes only. These items are sold without warranties of any kind.

(3) RoHS values:  Yes, No, RoHS Exempt. See the TI RoHS Statement for additional information and value definition.

(4) Lead finish/Ball material: Parts may have multiple material finish options. Finish options are separated by a vertical ruled line. Lead finish/Ball material values may wrap to two lines if the finish value exceeds the maximum column width.

(5) MSL rating/Peak reflow: The moisture sensitivity level ratings and peak solder (reflow) temperatures. In the event that a part has multiple moisture sensitivity ratings, only the lowest level per JEDEC standards is shown. Refer to the shipping label for the actual reflow temperature that will be used to mount the part to the printed circuit board.

(6) Part marking:  There may be an additional marking, which relates to the logo, the lot trace code information, or the environmental category of the part.

Multiple part markings will be inside parentheses. Only one part marking contained in parentheses and separated by a "~" will appear on a part. If a line is indented then it is a continuation of the previous line and the two combined represent the entire part marking for that device.

Important Information and Disclaimer:The information provided on this page represents TI's knowledge and belief as of the date that it is provided. TI bases its knowledge and belief on information provided by third parties, and makes no representation or warranty as to the accuracy of such information. Efforts are underway to better integrate information from third parties. TI has taken and continues to take reasonable steps to provide representative and accurate information but may not have conducted destructive testing or chemical analysis on incoming materials and chemicals. TI and TI suppliers consider certain information to be proprietary, and thus CAS numbers and other limited information may not be available for release.

In no event shall TI's liability arising out of such information exceed the total purchase price of the TI part(s) at issue in this document sold by TI to Customer on an annual basis.

Addendum-Page 2

www.ti.com	24-Jul-2025

##### TAPE AND REEL INFORMATION

Reel Diameter

K0	P1

B0

W

Cavity

A0

REEL DIMENSIONS	TAPE DIMENSIONS

| A0 | Dimension designed to accommodate the component width |
| --- | --- |
| B0 | Dimension designed to accommodate the component length |
| K0 | Dimension designed to accommodate the component thickness |
| W | Overall width of the carrier tape |
| P1 | Pitch between successive cavity centers |

Reel Width (W1)

QUADRANT ASSIGNMENTS FOR PIN 1 ORIENTATION IN TAPE

Q1

Q2

Q3

Q4

| Q1 | Q2 |
| --- | --- |
| Q3 | Q4 |

Sprocket Holes

User Direction of Feed

Pocket Quadrants

*All dimensions are nominal

| Device | Package Type | Package Drawing | Pins | SPQ | Reel Diameter (mm) | Reel Width W1 (mm) | A0 (mm) | B0 (mm) | K0 (mm) | P1 (mm) | W (mm) | Pin1 Quadrant |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TLC7524CDR | SOIC | D | 16 | 2500 | 330.0 | 16.4 | 6.5 | 10.3 | 2.1 | 8.0 | 16.0 | Q1 |
| TLC7524CNSR | SOP | NS | 16 | 2000 | 330.0 | 16.4 | 8.1 | 10.4 | 2.5 | 12.0 | 16.0 | Q1 |
| TLC7524CPWR | TSSOP | PW | 16 | 2000 | 330.0 | 12.4 | 6.9 | 5.6 | 1.6 | 8.0 | 12.0 | Q1 |
| TLC7524EDR | SOIC | D | 16 | 2500 | 330.0 | 16.4 | 6.5 | 10.3 | 2.1 | 8.0 | 16.0 | Q1 |
| TLC7524IDR | SOIC | D | 16 | 2500 | 330.0 | 16.4 | 6.5 | 10.3 | 2.1 | 8.0 | 16.0 | Q1 |
| TLC7524IPWR | TSSOP | PW | 16 | 2000 | 330.0 | 12.4 | 6.9 | 5.6 | 1.6 | 8.0 | 12.0 | Q1 |

### PACKAGE MATERIALS INFORMATION

www.ti.com	24-Jul-2025

TAPE AND REEL BOX DIMENSIONS

Width (mm)

*All dimensions are nominal

| Device | Package Type | Package Drawing | Pins | SPQ | Length (mm) | Width (mm) | Height (mm) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| TLC7524CDR | SOIC | D | 16 | 2500 | 350.0 | 350.0 | 43.0 |
| TLC7524CNSR | SOP | NS | 16 | 2000 | 353.0 | 353.0 | 32.0 |
| TLC7524CPWR | TSSOP | PW | 16 | 2000 | 353.0 | 353.0 | 32.0 |
| TLC7524EDR | SOIC | D | 16 | 2500 | 350.0 | 350.0 | 43.0 |
| TLC7524IDR | SOIC | D | 16 | 2500 | 350.0 | 350.0 | 43.0 |
| TLC7524IPWR | TSSOP | PW | 16 | 2000 | 353.0 | 353.0 | 32.0 |

Pack Materials-Page 2

www.ti.com	24-Jul-2025

##### TUBE

T - Tube

height	L - Tube length

W - Tube width

B - Alignment groove width

*All dimensions are nominal

| Device | Package Name | Package Type | Pins | SPQ | L (mm) | W (mm) | T (µm) | B (mm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| TLC7524CD | D | SOIC | 16 | 40 | 505.46 | 6.76 | 3810 | 4 |
| TLC7524CD.A | D | SOIC | 16 | 40 | 505.46 | 6.76 | 3810 | 4 |
| TLC7524CDG4 | D | SOIC | 16 | 40 | 505.46 | 6.76 | 3810 | 4 |
| TLC7524CN | N | PDIP | 16 | 25 | 506 | 13.97 | 11230 | 4.32 |
| TLC7524CN.A | N | PDIP | 16 | 25 | 506 | 13.97 | 11230 | 4.32 |
| TLC7524CNE4 | N | PDIP | 16 | 25 | 506 | 13.97 | 11230 | 4.32 |
| TLC7524CNS | NS | SOP | 16 | 50 | 530 | 10.5 | 4000 | 4.1 |
| TLC7524CNS.A | NS | SOP | 16 | 50 | 530 | 10.5 | 4000 | 4.1 |
| TLC7524CPW | PW | TSSOP | 16 | 90 | 530 | 10.2 | 3600 | 3.5 |
| TLC7524CPW.A | PW | TSSOP | 16 | 90 | 530 | 10.2 | 3600 | 3.5 |
| TLC7524ED | D | SOIC | 16 | 40 | 505.46 | 6.76 | 3810 | 4 |
| TLC7524ED.A | D | SOIC | 16 | 40 | 505.46 | 6.76 | 3810 | 4 |
| TLC7524EN | N | PDIP | 16 | 25 | 506 | 13.97 | 11230 | 4.32 |
| TLC7524EN.A | N | PDIP | 16 | 25 | 506 | 13.97 | 11230 | 4.32 |
| TLC7524ID | D | SOIC | 16 | 40 | 505.46 | 6.76 | 3810 | 4 |
| TLC7524ID.A | D | SOIC | 16 | 40 | 505.46 | 6.76 | 3810 | 4 |
| TLC7524IN | N | PDIP | 16 | 25 | 506 | 13.97 | 11230 | 4.32 |
| TLC7524IN.A | N | PDIP | 16 | 25 | 506 | 13.97 | 11230 | 4.32 |
| TLC7524IPW | PW | TSSOP | 16 | 90 | 530 | 10.2 | 3600 | 3.5 |
| TLC7524IPW.A | PW | TSSOP | 16 | 90 | 530 | 10.2 | 3600 | 3.5 |

# FN0020A

PACKAGE OUTLINE

#### PLCC - 4.57 mm max height

SCALE 1.300

PLASTIC CHIP CARRIER

B	.350-.356

[8.89-9.04]

NOTE 3

A	3	1	20

.180 MAX

[4.57]

(.008)

[0.2]

.020   MIN

[0.51]

4	18

.350-.356

[8.89-9.04]

NOTE 3

PIN 1 ID (OPTIONAL)

.283-.339

[7.19-8.61]

8	14

20X .026-.032

[0.66-0.81]

9	13

.090-.120

[2.29-3.04]

TYP

C

20X .013-.021

[0.33-0.53]

|  | .007 [0.18] | C | A | B |
| --- | --- | --- | --- | --- |

16X

SEATING PLANE

.050

[1.27]

.004 [0.1]  C

.385-.395

[9.78-10.03] TYP

4215152/B  04/2017

NOTES:

- All linear dimensions are in inches. Any dimensions in brackets are in millimeters. Any dimensions in parenthesis are for reference only. Controlling dimensions are in inches. Dimensioning and tolerancing per ASME Y14.5M.

- This drawing is subject to change without notice.

- Dimension does not include mold protrusion. Maximum allowable mold protrusion .01 in [0.25 mm] per side.

- Reference JEDEC registration MS-018.

# FN0020A

## EXAMPLE BOARD LAYOUT

#### PLCC - 4.57 mm max height

PLASTIC CHIP CARRIER

20X (.096  )

[2.45]

3

SYMM

1	20

(R.002 ) TYP [0.05]

20X (.025  )

[0.64]	4

18

SYMM

(.327)

[8.3]

16X (.050  )

[1.27] 	 14

8

9	13

(.327)

[8.3]

LAND PATTERN EXAMPLE

EXPOSED METAL SHOWN SCALE:6X

.002   MAX

[0.05]

EXPOSED METAL

.002   MIN

[0.05]

EXPOSED METAL

ALL AROUND

ALL AROUND

METAL	SOLDER MASK OPENING

NON SOLDER MASK DEFINED (PREFERRED)

SOLDER MASK OPENING

SOLDER MASK DEFINED

METAL UNDER SOLDER MASK

###### SOLDER MASK DETAILS

4215152/B  04/2017

NOTES: (continued)

- Publication IPC-7351 may have alternate designs.

- Solder mask tolerances between and around signal pads can vary based on board fabrication site.

# FN0020A

## EXAMPLE STENCIL DESIGN

#### PLCC - 4.57 mm max height

PLASTIC CHIP CARRIER

20X (.096  )

[2.45]

SYMM

3	1	20

(R.002 ) TYP [0.05]

20X (.025  )

[0.64]	4

18

SYMM

(.327)

[8.3]

16X (.050  )

[1.27] 	 14

8

9	13

(.327)

[8.3]

###### SOLDER PASTE EXAMPLE

BASED ON 0.125 mm THICK STENCIL SCALE:6X

4215152/B  04/2017

NOTES: (continued)

- Laser cutting apertures with trapezoidal walls and rounded corners may offer better paste release. IPC-7525 may have alternate design recommendations.

- Board assembly site may have different recommendations for stencil design.

# NS0016A

SCALE 1.500

## PACKAGE OUTLINE

#### SOP - 2.00 mm max height

SOP

TYP

| A |  |  |
| --- | --- | --- |
| 1 |  |  |

8.2

1.27

7.4

C

SEATING PLANE

PIN 1 ID AREA

14X

16

0.1 C

10.4

10.0

NOTE 3

2X

8.89

8

9

16X 0.51

B

5.4

5.2

NOTE 4

0.35

|  | 0.25 | C | A | B |
| --- | --- | --- | --- | --- |

2.00 MAX

0.15 TYP

SEE DETAIL A

0.25

GAGE PLANE

0.3

0.1

0  - 10

1.05

0.55

(1.25)

DETAIL A

TYPICAL

4220735/A  12/2021

NOTES:

- All linear dimensions are in millimeters. Dimensions in parenthesis are for reference only. Dimensioning and tolerancing per ASME Y14.5M.

- This drawing is subject to change without notice.

- This dimension does not include mold flash, protrusions, or gate burrs. Mold flash, protrusions, or gate burrs shall not exceed 0.15 mm, per side.

- This dimension does not include interlead flash. Interlead flash shall not exceed 0.25 mm, per side.

# NS0016A

## EXAMPLE BOARD LAYOUT

#### SOP - 2.00 mm max height

SOP

16X (1.85)

SYMM

SEE DETAILS

1	16

16X (0.6)

SYMM

14X (1.27)

8	9

(R0.05) TYP

(7)

LAND PATTERN EXAMPLE

SCALE:7X

METAL

SOLDER MASK OPENING

SOLDER MASK	METAL OPENING

0.07  MAX ALL AROUND

0.07 MIN

ALL AROUND

NON SOLDER MASK DEFINED

SOLDER MASK DEFINED

SOLDER MASK DETAILS

4220735/A  12/2021

NOTES: (continued)

- Publication IPC-7351 may have alternate designs.

- Solder mask tolerances between and around signal pads can vary based on board fabrication site.

# NS0016A

## EXAMPLE STENCIL DESIGN

#### SOP - 2.00 mm max height

SOP

16X (1.85)

SYMM

1	16

16X (0.6)

SYMM

14X (1.27)

8	9

(R0.05) TYP

(7)

SOLDER PASTE EXAMPLE

BASED ON 0.125 mm THICK STENCIL SCALE:7X

4220735/A  12/2021

NOTES: (continued)

- Laser cutting apertures with trapezoidal walls and rounded corners may offer better paste release. IPC-7525 may have alternate design recommendations.

- Board assembly site may have different recommendations for stencil design.

# PW0016A

## PACKAGE OUTLINE

#### TSSOP - 1.2 mm max height

SCALE 2.500

SMALL OUTLINE PACKAGE

TYP

6.6

A	6.2

SEATING PLANE

C

0.1 C

PIN 1 INDEX AREA

1

14X

0.65

16

5.1

4.9

NOTE 3

2X

4.55

4X (0  -12 )

8

B

4.5

4.3

NOTE 4

9	0.30

16X

|  | 0.1 | C | A | B |
| --- | --- | --- | --- | --- |

0.17

1.2 MAX

(0.15) TYP

SEE DETAIL A

0.25

GAGE PLANE

0.15

0.05

0 -8

0.75

0.50

DETA  20 AIL A

TYPICAL

4220204/B  12/2023

NOTES:

- All linear dimensions are in millimeters. Any dimensions in parenthesis are for reference only. Dimensioning and tolerancing per ASME Y14.5M.

- This drawing is subject to change without notice.

- This dimension does not include mold flash, protrusions, or gate burrs. Mold flash, protrusions, or gate burrs shall not exceed 0.15 mm per side.

- This dimension does not include interlead flash. Interlead flash shall not exceed 0.25 mm per side.

- Reference JEDEC registration MO-153.

# PW0016A

## EXAMPLE BOARD LAYOUT

#### TSSOP - 1.2 mm max height

SMALL OUTLINE PACKAGE

1

16X (0.45)

16X (1.5)

SYMM

(R0.05) TYP

16

SYMM

14X (0.65)

8	9

(5.8)

LAND PATTERN EXAMPLE

EXPOSED METAL SHOWN SCALE: 10X

SOLDER MASK

OPENING

METAL

METAL UNDER SOLDER MASK

SOLDER MASK OPENING

EXPOSED METAL	 		 		EXPOSED METAL

0.05  MAX ALL AROUND

0.05 MIN

ALL AROUND

NON-SOLDER MASK DEFINED (PREFERRED)

SOLDE15.000 R MASK DETAILS

SOLDER MASK DEFINED

NOTES: (continued)

- Publication IPC-7351 may have alternate designs.

- Solder mask tolerances between and around signal pads can vary based on board fabrication site.

4220204/B  12/2023

# PW0016A

## EXAMPLE STENCIL DESIGN

#### TSSOP - 1.2 mm max height

SMALL OUTLINE PACKAGE

1

16X (0.45)

16X (1.5)

SYMM

(R0.05) TYP

16

SYMM

14X (0.65)

8	9

(5.8)

SOLDER PASTE EXAMPLE

BASED ON 0.125 mm THICK STENCIL SCALE: 10X

4220204/B  12/2023

NOTES: (continued)

- Laser cutting apertures with trapezoidal walls and rounded corners may offer better paste release. IPC-7525 may have alternate design recommendations.

- Board assembly site may have different recommendations for stencil design.

IMPORTANT NOTICE AND DISCLAIMER

TI PROVIDES TECHNICAL AND RELIABILITY DATA (INCLUDING DATASHEETS), DESIGN RESOURCES (INCLUDING REFERENCE DESIGNS), APPLICATION OR OTHER DESIGN ADVICE, WEB TOOLS, SAFETY INFORMATION, AND OTHER RESOURCES “AS IS” AND WITH ALL FAULTS, AND DISCLAIMS ALL WARRANTIES, EXPRESS AND IMPLIED, INCLUDING WITHOUT LIMITATION ANY IMPLIED WARRANTIES OF MERCHANTABILITY, FITNESS FOR A PARTICULAR PURPOSE OR NON-INFRINGEMENT OF THIRD PARTY INTELLECTUAL PROPERTY RIGHTS.

These resources are intended for skilled developers designing with TI products. You are solely responsible for (1) selecting the appropriate TI products for your application, (2) designing, validating and testing your application, and (3) ensuring your application meets applicable standards, and any other safety, security, regulatory or other requirements.

These resources are subject to change without notice. TI grants you permission to use these resources only for development of an application that uses the TI products described in the resource. Other reproduction and display of these resources is prohibited. No license is granted to any other TI intellectual property right or to any third party intellectual property right. TI disclaims responsibility for, and you fully indemnify TI and its representatives against any claims, damages, costs, losses, and liabilities arising out of your use of these resources.

TI’s products are provided subject to TI’s Terms of Sale, TI’s General Quality Guidelines, or other applicable terms available either on ti.com or provided in conjunction with such TI products. TI’s provision of these resources does not expand or otherwise alter TI’s applicable warranties or warranty disclaimers for TI products. Unless TI explicitly designates a product as custom or customer-specified, TI products are standard, catalog, general purpose devices.

TI objects to and rejects any additional or different terms you may propose.

Copyright © 2025, Texas Instruments Incorporated Last updated 10/2025